| 7 Pin CONNECTOR |  |  |
| --- | --- | --- |
| CIRCUIT | CIRCUIT CONDUCTOR | COLOUR |
| 1 | Left-Hand Turn | Yellow |
| 2 | Not used (designated for reverse) | Black |
| 3 | Earth Return | White |
| 4 | Right-Hand Turn | Green |
| 5 | Service Brakes | Blue |
| 6 | Stop Lamps | Red |
| 7 | Rear Lamps, Clearance Lamps | Brown |

Wiring Code   REDLINE TRAILERS    as per VSB1 v5  Effective from   5/4/2018

| 2 Pin Weather Pack Connector |  |  |
| --- | --- | --- |
| Circuit | Circuit Conductor | Colour |
| A | AUX (Breakaway Battery Charge) | Pink |
| B | Battery Monitor as per VIB6 (NSW/ACT only) | Violet |
| 12 Pin CONNECTOR |  |  |
| Circuit | Circuit Conductor | Colour |
| 1 | Left-hand turn | Yellow |
| 2 | Not used (designated for reverse) | Black |
| 3 | Earth Return(> Circuit Pins 1,4,5,6,7,12, A,B ) | White |
| 4 | Right-hand turn | Green |
| 5 | Service brakes | Blue |
| 6 | Stop lamps | Red |
| 7 | Rear lamps, clearance lamps | Brown |
| 8 | Battery (+ 12/24 v ) > Feed to > BCDC | Orange |
| 9 | AUX (+12v) Ignition > Brake safe charge | Pink |
| 10 | Earth return (-) > ( Circuit Pins 8,9 &11 ) | White |
| 11 | Accessories ( + 12/24 v ) > BCDC | Grey |
| 12 | Battery monitor as per VIB6 (NSW/ACT only ) | Violet |

Notes		Circuit Pins 3 & 10 maybe linked at vehicle (20 Amp)

Circuit Pin 9  12 volt only

Installations of 12 volt only Pin 11 & 9 may be linked at vehicle (10 Amp)