[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

WORKING WITH HYDRAULICS
PPE (Minimum – check SWP- PPE for correct protection)

· Long hair secured

· Eye Protection

· Safety Footwear

· Gloves

OPERATION
· Shut down machine. 
· Attach Danger Tags. (Isolation)
· Place all hydraulic implements on ground or on stands if necessary.

· When using Hydraulic implements to jack up machinery, ensure that adequate safety stands are placed in suitable positions under the vehicle to safely hold the vehicles weight, once stands are in position make sure the Hydraulic pressure is released to rest all vehicle weight on the stands.

NOTE: Pressure can be trapped in hoses, so care must be taken. When removing Hydraulic hoses follow the precautions below.
· Release all hydraulic pressures at controls.
· Have spill kits/drain trays in case of spill.
· Proceed to work. Don’t pull tooling towards body or head if excess force required. 
· Do not check for hydraulic leaks with your hands – see hazards.

· Release hydraulic tank pressure from cap or breather.

NOTE:  Don’t assume Hydraulic pressure is released. 

             E.g. keep in mind most hydraulic circuits have accumulators.

HAZARDS
· Falling implements i.e. Bucket/blade.
· Injury from slipping tooling, e.g., pinch bar, wrenches.
· Hydraulic oil injection into skin or eyes.
· Machinery falling from incorrectly positioned safety stands.
· Environmental damage from spilt fluid.
[image: image1.png]Approved by Manager/Supervisor:                                                 






Date:       /       /2016
Dieselforce
6.13 Working With Hydraulics
Revision: 2. 8th October 2015

