[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

OXY - ACETYLENE EQUIPMENT
PPE (Minimum – check SWP- PPE for correct protection)

Long hair secured

Eye Protection

Safety Footwear

Welding Gloves

Welding Mask
Welding Screen (to prevent others from getting welding flashes)

Long sleeved shirts - buttoned
OPERATION
· Check equipment: condition of bottles (and date stamp), hand piece, hoses. Do not use defective equipment. 
· Cylinders to be secured in upright position while being transported or used.
· Ensure flash arrestors fitted to bottles and hand piece.

· Ensure fire extinguishers are accessible.
· Ensure adequate ventilation.

· Remove all flammable materials – 12m radius.

· Turn on taps to bottles (1/4 to ½ turn).

· Set correct pressures on regulators.

· Start with acetylene and add oxygen to set flame.

· When job is complete turn off acetylene first, oxygen second.

· Turn bottles off after each use.

HAZARDS
· In the event of flash back. Turn off hand piece oxygen valve first and acetylene second. NOTE: This is the reverse of the normal shutting down procedure
· Fire

· Burns

· Incorrect fittings: LPG –v- Acetylene
· Ultra violet light 

· Frayed material on clothing

· Perished hoses

· Broken or ill operating hand pieces

[image: image1.png]Approved by Manager/Supervisor:                                                 






Date:       /       / 2016
Dieselforce
6.14  Oxy – Acetylene Equipment
Revision: 1. 20th June 2013

