[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

DEGASSING AIR CON SYSTEM

Note:   Air con recovery to be performed by ticketed 134A personnel only

PPE (Minimum – check SWP- PPE for correct protection)
Long hair secured


Long sleeve buttoned
Eye protection
No loose clothing
OPERATION
· Check gas recovery bottle for leakage and in-date.
· If out of date, or unable to stop leak, cap bottle and return to supplier.
· Fit gauges low and high to correct points.
· Weigh bottle on scales and record weight.
· Connect recovery bottle to recovery pump unit (read instruction of re-connecting pump unit if unsure of operation).
· Open low pressure gauge, check for leaks (rectify if leak found on bottle or unit).
· Open recovery bottle, check for leaks (if leak found, rectify leak).

· Recover air con gas 134A only.
· At completion of recovery, turn off all taps.
· Turn off machine.
· Turn off all taps on equipment.
· Weigh bottle and record weight.
· Carry out repairs to air con system as required.
HAZARDS

· Environment Damage

· Refrigerant burn from accidental discharge
[image: image1.png]Approved by Manager/Supervisor:                                                 







Date:       /       / 2016
Dieselforce
6.18 Degassing Air Con System
Revision: 1. 19th September 2012

