[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

RE-GASSING OF AIR CON SYSTEM WITH 134A GAS

Note: Only to be carried out by ticketed person to handle 134A gas

PPE (Minimum – check SWP- PPE for correct protection)
Long hair secured
Long sleeves buttoned
Eye protection
No loose clothing
OPERATION
· Bottle to be checked for leaks with detector before use, cap to be fitted.
· Weigh bottle before use and record weight.
· Connect hosing.
· Turn bottle on, check for leaks prior to use, if leaks found, rectify

· Add liquid to Hi side of system. Note:  if weight for system is available fill to below weighted system.
· Operate vehicle air con system.
· Charge system through low side with gas only, checking weight of bottle.
· On completion of charging system, record weight used.

· Fill out air con sticker with weight, date, etc. Fix to suitable place on vehicle.
· Remove gauges, check bottle and charge points for leaks. Rectify any leaks, fit caps.
HAZARDS

· Environmental Damage

· Refrigerant burn from accidental discharge

[image: image1.png]Approved by Manager/Supervisor:                                                 







Date:       /       /  2016
Dieselforce
6.19  Re-gassing Air Con System with 134A Gas
                 Revision: 1. 10th October 2012

