
[image: image2.png]et OSEE OSEE OSNEN OSNEN OGN OSNENI O,

DIESEL'FORCE

‘-“-‘V





SAFE WORK PRACTICE

BATTERY CHARGING
PPE (Minimum – check SWP- PPE for correct protection)

Long sleeves buttoned

Eye protection-face shield
PVC gloves
Safety footwear

Refer to MSDS for options
OPERATION

· Check eye wash is operating correctly

· No Smoking Policy must be observed

· Ensure batteries have maximum ventilation
· DO NOT connect leads if strong hydrogen smell present – ventilate area first

· Remove battery plugs before charging

· Ensure cells are topped up
· Ensure charger is set on correct voltage 

· Power supply must be off before connection

· Check vents on cell plugs are clear before replacing

· Power supply must be off before disconnection
· Wash down any spillage

· Leave area tidy

HAZARDS
 Refer to MSDS for complete list
· Severe eye, skin and respiratory tract damage from highly corrosive liquid and or fumes.

· High risk of fire and explosion from readily detonated hydrogen gases built up during charging process; over charging produces excess gases and hydrogen mists.
[image: image1]









Approved by Manager/Supervisor:                                                 






Date:       /       /   2016
Diesel Force
6.2 Battery Charging
Revision: 1. 24th April 2014

[image: image2.png]