[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

WASTE OIL SEPARATOR 
PPE (Minimum – check SWP- PPE for correct protection)
Long hair secured
Long sleeves buttoned 
Safety Glasses

Gloves (PVC)
OPERATION
· Weekly
· Inspect wash pad drain and clean out if necessary.

· Drain sludge from oil separator.

· Ensure all manual valves are working.

· Remove lid on separator and inspect comb filter. Clean if required.

· Ensure catchments at front and rear of workshop are free of oil in the event of heavy rainfall.

· Remove sump cover and inspect depth of sump if more than one foot deep will need to be cleaned out.

Monthly

· Complete weekly inspection.

· Remove sump cover and manually operate float valve to ensure system is working. 

HAZARDS

Environmental Damage
EMERGENCY/MALFUNCTION
Turn off pump immediately.

Contain any overflow.

Report to supervisor to have repairs carried out.

[image: image1.png]Approved by Manager/Supervisor:                                                 







Date:       /       / 2016
Dieselforce
6.21  Waste Oil Separator
Revision: 1. 24thh March 2011

