[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

405mm CUT OFF SAW 
Under no circumstances are non-ferrous metals to be cut on this saw e.g. Aluminium
Do not use side of cutting disc as a grinder, it could shatter.
PPE (Minimum – check SWP- PPE for correct protection)
Long hair secured
Long sleeves buttoned

Hearing protection 
No loose clothing
Double Eye protection 

-
Safety Glasses and Full Face Shield (primary and secondary).

-
Welding helmets NOT to be used as face shields.
OPERATION
· Turn lights on over work area.
· Ensure guards are in place.
· Ensure machine is in serviceable condition

· Clamp work into vice provided (use stand to support extra steel).
· Do not cut chain with cut off saw.

· Make sure cut off side of grinder is clear of all obstacles when metal drops e.g. power cords on the ground. 
· Hold on main handle on start up to avoid torque jolt.
· Proceed to cut allowing the disc to do the work (don’t force it).
· Turn off saw.
· Unplug saw if wheels require changing.

· Remove work (be careful of hot surfaces).
· Leave area tidy.
· Turn off lights.
HAZARDS

· Burns
· Cuts

· Eye injury

· Hearing damage

· Electric shock

· Serious injury from unsecured work/disc shattering

[image: image1.png]Approved by Manager/Supervisor:                                                 







Date:       /       /   2016
Dieselforce
6.22  Cut Off Saw
Revision: 1. 2nd May 2012

