[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

PRESSURE WASHER CLEANER
PPE (Minimum – check SWP- PPE for correct protection)
Long hair secured
Eye Protection

Gloves (optional)
Face Shield
OPERATION
· Before washing you may require the use of our degreaser/detergent. If you experience skin reactions, wear gloves.
· When using pressure washer gun hold it with both hands.

· Turn off pressure washer at switch when finished.

· Do not let auto shut it down. Can wear out pressure washer.
· Wash soiled parts on wash pad area.

· Do not hold or secure items with body parts while washing items.

· All vehicles to be washed in the designated areas (wash pad or alley way drain).

· Use face shield in confined areas where blowback can occur.

· Do not direct pressure stream at personnel.

· Do not direct pressure stream into electrical parts or seals on vehicles.
· Be aware that pressure washer can remove decals from vehicles Keep pressure stream 500 mm away from decals when washing.
· Ensure pressure washer is turned off after use.
· When washing customer’s vehicles have consideration for paint work and don’t get too close.

· If using broom to scrub down, ensure it is clean before use.

HAZARDS

· Skin injuries from high pressure

· Eye injuries from blow back

· Skin irritation from chemicals
· Environmental Damage
[image: image1.png]Approved by Manager/Supervisor:                                                 







Date:       /       /   2016
Dieselforce
6.24  Pressure Washer Cleaner
Revision: 1. 22nd Feb. 2012

