[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

FIBREGLASSING
Application

To provide a standard work practice for Fibreglassing work, as required by legislation and in accordance with the Diesel Force safety system.

Fibreglass Resin is classified as a hazardous substance according to criteria of Worksafe Australia.

1. Personal Protective Equipment / Special Equipment

· protective clothing

· safety footwear

· eye protection-goggles and face shield
· Respiratory protection – half face filter respirator with appropriate organic vapour / particulate cartridge if ventilation not adequate.
· Solvent Resistant gloves

· Ensure all skin areas are completely covered

2. Requirements

2.1 Fibreglass Workers Precautions

· Before commencing work, read Fibreglass Resin MSDS for First Aid requirements.  Follow relevant MSDS information for safe handling and clean-up of Fibreglass Resin.

· Ensure there will be adequate natural ventilation at all times, prior to commencement of work.
· Ensure respiratory equipment and protective clothing is worn at all times during Fibreglassing operations.  Seek specialist advice if unsure.

· Be aware of the location of portable fire extinguishers in case of fire, and ensure they are accessible.
· Should an emergency arise, follow the Emergency Response Plan.

· Advise all relevant personnel when Fibreglassing is about to commence and the anticipated duration of the work.

· Do not empty into drains.

· Before eating, drinking or smoking, remove contaminated clothing and wash hands.

Hazards

· Damage to respiratory tract and central nervous system (dizziness, nausea, euphoria, vomiting, loss of coordination, drowsiness).
· Asphyxiation

· Dermatitis from repeated skin contact

· Handling flammable materials, Vapours are heavier than air and can travel considerable distance to an ignition source
· Environmental and equipment damage


Approved by Manager/Supervisor:                                                 







Date:       /       /   2016 
Diesel Force
6.25 Fibreglassing
Revision: 0. 21st July 2016

[image: image1.png]