[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

LIFTING TRUCK CAB UP
PPE (Minimum – check SWP- PPE for correct protection)
· Long hair secured
· Safety glasses
OPERATION
· Lower bull bar before tipping cab and pop bonnets if required.  
· Remove all items from inside cab that could fall and break windscreen.
· Lift cab as per manufacturer’s procedure. If heavy, use two people to lift.

· Inspect security of rams and/or locking devices.

· When cab is up, secure into place with pin provided or make a support.

· When job is complete, remove pin or support and lower cab.
· If two required to raise cab, use two to lower.

· Ensure cab locks are activated. 

· Secure cab and replace all items to their original position. Tighten bull bar.
HAZARDS

· Personal injury from heavy lifting
· Personal injury from cab coming down

· Personal injury from cab moving during operation of truck

· Broken windscreen

[image: image1.png]Approved by Manager/Supervisor:                                                 







Date:       /       /   2016  

Diesel Force
6.28 Lifting Truck Cab Up
Revision: 3. 21st June 2013

