[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

CHECKING SUSPENSION

PPE (Minimum – check SWP- PPE for correct protection)
· Long hair secured

· Long sleeves buttoned

· Eye protection

· No loose clothing

OPERATION
· Complete suspension check as per service sheet checklist.
· Look for signs of Rust, Fretting and Metal Fatigue.

· All bolts and U-bolts for suspension to be checked with hammer for loose fit.

· Check for sideways movement in spring pin hangers and bushes.
· Don’t pull tooling towards body or head if excess force required. 

If unsure of anything whilst checking see workshop supervisor for second opinion before sending vehicle back to client.

HAZARDS
· Pinch points – eye injury – projectiles

· Injury from slipping tooling, e.g., pinch bar, wrenches.

· Air Bags (don’t crawl under without chassis supports)

· Be aware how air suspension works.

[image: image1.png]Approved by Manager/Supervisor:                                                 







Date:       /       /2016  

Dieselforce
6.29  Checking Suspension
Revision: 1. 13th Sept. 2013

