[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

TEST AIR CON FOR LEAKS - NITROGEN
PPE (Minimum – check SWP- PPE for correct protection)
· Long hair secured

· Long sleeves buttoned

· Eye protection

· No loose clothing
OPERATION
· Clear personnel from area.

· Connect nitrogen.

· Bring to max 350 psi.

· Bring up in stages. 
· Check worn/old hosing for leaks and condition.

· Check front seal of compressor.

· Old or perished hosing could fail under pressure.

· Ensure bottle is secured on trolley and tied off correctly.

HAZARDS

· High pressures
· Eye injury from burst hose

· Bottle falling over

· Air con system damage from over pressure

[image: image1.png]Approved by Manager/Supervisor:                                                 







Date:     /    /  2016
Dieselforce
6.31 Test Air Con for Leaks
Revision: 2. 10th April 2014

