[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

MAGNETIC BASE DRILL
PPE (Minimum – check SWP- PPE for correct protection)

· Long hair secured.
· Long sleeves buttoned.
· Eye protection.
· No loose clothing.
· No Gloves.
OPERATION

· Turn lights on over work area.
· Ensure Magnetic Base drill is secured when mounted vertical. (Use sling and forklift if needed).

· Ensure mounting surface is clean and flat to allow maximum magnetic power for drill stability.

· Ensure that mounting surface is in fact magnetic (Not stainless or aluminium.)

· Fit Drill bit or Broach required for the job, remembering to remove the chuck key before continuing. 
· Ensure cutting fluid is used with rota broach bits. 
· Follow MSDS requirements for cutting fluid.
· Proceed to work.
· Ensure that downward pressure does not force the magnet to release from the steel surface.

· Do not allow swarf to exceed 100mm in length, remove it by lifting the drill bit out of the work so it breaks off before continuing.
· Never leave drill un-attended for long periods of time in vertical operation.

· Turn rotation off before magnet.
· Remove Drill bit or Broach from work switch off and unplug drill.
· Clean drill and leave area tidy.
· Switch off lights.
 HAZARDS

· Failing to remove chuck key may cause serious injury.
· Swarf longer than 100mm in length may cause serious injury.
· Power Failure and magnet turns off; Drill can fall onto operator.

[image: image1.png]Approved by Manager/Supervisor:                                                 







Date:       /       /   2016  

Dieselforce
6.37 Magnetic Base Drill
Revision: 1. 8th August 2012

