[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

MEGACHLOR
To be used by properly trained personnel only
Refer to MSDS before use

PPE (Minimum – check SWP- PPE for correct protection)

· Long hair secured

· Safety Footwear

· Gloves

· Safety glasses and face shield
· Long sleeved shirts – buttoned
STORAGE
· Keep containers closed
DILUTION

· 3 Litres of Megachlor to 95 Litres of water (One complete fill of the dispenser) 
OPERATION

· Ensure adequate ventilation.

· Avoid spillage

· Mix with water only

· Close containers after each use

· Diluted product to be disposed of through sewer system

HAZARDS
· Vapour may be harmful.
· Can cause burns to mouth, eyes and skin. Treat with water.
· May give off dangerous gas if mixed with other chemicals.
[image: image1.png]Approved by Manager/Supervisor:                                                 






Date:      /    /   2016
Dieselforce
6.41  Megachlor
Revision: 1. 31st Oct. 2012

