SAFE WORK PRACTICE

USE AND MAINTENANCE OF SLINGS

NEVER STAND or WALK UNDER A LOAD

NEVER Adjust a sling as it is being tightened

PPE (Minimum – check SWP- PPE for correct protection)

- Long hair secured

- Safety Footwear

- Gloves

- Safety glasses

- Hard hat if load is being lifted overhead

STORAGE – round (fibre) slings

- Only use round slings when necessary, e.g., when possible damage to chrome rods or paint work could occur.

- All attempts should be made to keep slings free of oil, grease and other chemicals.

- Tag the new sling with the current tag when put into operation.

OPERATION – do not use wire slings

The following applies to both round (fibre) slings and chain slings

ALWAYS do the following

- Select the right sling and attachments for the lift.

- Inspect each item of lifting equipment before attachment to the load.

- Balance the load to avoid overstress on any legs of the sling.

- Protect slings from damage by padding sharp edges with corner saddles or wooden blocks

- Hook unused slings to the oblong link or sling rings

- Keep hands and fingers away when slack is being taken out of a sling

- Tag ‘Out of Service’ if sling damaged, oily or tag missing

1 of 2

NEVER do any of the following:

- Drag slings/chains from under a load or across floors.

- Drop slings/chains from a height.

- Use bolts or shackles for joining or shortening a sling/chain.

- Shorten a sling/chain with knots.

- Use chains with links worn more than 10%.

- Use any sling without the approved SWL tag attached.

- Hammer a sling into place.

- Carry a load on the point of a hook, or insert the point of the hook into a link of the chain

- Adjust a sling/chain as it is being tightened

MAINTENANCE

Round slings – before use:

Visually inspect the sling before use and if any of the following are evident, discard the sling, obtain a new sling and book to the job;

- Damaged or worn in any way.

- Dirty or oily. Oil and chemicals can weaken the sling.

- The tag is out of date or missing. Tag ‘Out of Service’ and leave for inspection.

Chain slings - before use:

Check that the sling is tagged, carefully visually inspect the chain links, hook/s, master links, hammer locks and any other fittings and if any of the following are evident, tag the sling, remove it from service and notify your supervisor;

- Metal tag is missing or unreadable.

- Links are worn more than 10%.

- Links are elongated, bent, corroded or distorted.

- Obvious wear in any fitting

- Stretched hook, damaged safety latch or damaged self-locking hook.

HAZARDS

- Serious body injuries or death from slipping or dropped load.

- Hand injuries from pinch points.

- Injury from falling objects.

Approved by Manager/Supervisor:

Date:       /       /   2015

2 of 2