[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

VEHICLE SERVICE PIT
PPE (Minimum – check SWP- PPE for correct protection)

· Long hair secured

· Long sleeves buttoned

· Eye protection

· No loose clothing

· Gloves if needed
NEVER DRIVE A VEHICLE ONTO OR OFF THE PIT WITHOUT THE USE OF A DESIGNATED SPOTTER

OPERATION

· NEVER allow a customer to drive vehicles on or off the pit.

· Never reverse vehicles onto pit unless JSA done and approved by Supervisor.

· Ensure you have a clean work area before commencing.
· Align vehicle as straight as possible ready for entry onto pit.

· When guiding truck onto pit, spotter is to be at step until truck lined up, then exit pit and continue guiding truck onto pit from top of steps.

· Don’t stand under vehicle at any time while it is in motion.
· Spotter to ensure front wheels not to cross over onto the yellow marked edge.

· Driver is to take direction only from the designated spotter.

· Drive the vehicle onto the pit using the lowest gear possible.

· When vehicle in position ensure the park brake has been applied.

· Take care with foot position when using pit entry steps; don’t walk down steps without full vision.
· Turn on pit lighting if needed. Auto sensor should activate lights on entry
· When completing repairs take care not to hit head on under carriage.
· Ensure safety glasses are worn at all times to protect against falling debris.

· No trailers to be over pit

· Drain tins to be emptied immediately after service
· Reposition gate and barriers when vehicle moved off pit
 HAZARDS

· Vehicle falling into pit.

· Eye injury from falling debris

· Personnel falling into pit

· Head injury from undercarriage
[image: image1.png]



 Approved by Manager/Supervisor:                                                  








Date:       /     /   2016
Diesel Force
6.46  Vehicle Service Pit
Revision: 3. 18th April 2012

