SAFE WORK PRACTICE

#### LADDER SAFETY

PPE (Minimum – check SWP- PPE for correct protection)

- Long hair secured

- Long sleeves buttoned

- Eye protection

- No loose clothing

- Gloves If Needed

Ladders must be properly stored and inspected regularly and only used for light work of short duration or for access.

Before using a ladder, check that it:

- Has no damaged, loose or missing parts

- Is secured against movement and be supported from a firm, level, non-slip surface and/or there is another person holding the base of the ladder

- Projects at least 1 metre above the landing place

- Is placed at a slope that is no steeper than 4 units of height to 1 unit horizontally

- Is rated for industrial use, not domestic use.

When using a ladder:

- Always have two hands free to climb up and down (three points of contact)

- Any materials or tools (other than those held on a worker's belt) should be transferred to the work area separately

- Always face the ladder while climbing up, down or working

- Never place feet higher than 900mm from the top of the ladder

- Never over-reach from a ladder

- Never work from a ladder above another person

- Never have more than one person on a ladder at any one time

- Do not use a ladder in an access way or where it may be hit by a door

- Do not undertake work requiring restricted vision, welding or metal cutting from a ladder

- Use a non-metallic ladder where there are electrical hazards

- When the job is completed, store the ladder correctly.

Hazards

- Severe injury or death from falling.

- Contact with live electrical equipment.

Approved by Manager/Supervisor:

Date:       /       / 2016