[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





   SAFE WORK PRACTICE

HAND HELD DRILL
IMPORTANT

PPE (Minimum – check SWP- PPE for correct protection)

· Long hair secured

· Long sleeves buttoned
· Safety Glasses 
· Hearing protection.

· No loose clothing

· Gloves

OPERATION

· Turn lights on over work area 
· Ensure drill is unplugged before fitting or changing drill bits

· Ensure drill is NOT in locked on position, before you plug it in. 
· Fit drill bit required to do the job, remembering to remove the chuck key before continuing.
· Ensure drill bit is sharp and in good condition. 
· Position work and secure with drill clamp or vice.
· Under no circumstance is work to be held by hand.
· Check foot position for even force and resistance – 1 foot forward, 1 foot back.
· Make sure the electrical cord is away from the rotating head, put over shoulder if necessary.

· Use extra caution if it is necessary to drill above shoulder height.
· Use extra handle if using a large bit.
· Proceed to work.
· Keep constant, steady pressure on drill bit.

· Do not allow swarf to exceed 100mm in length, remove it by lifting the drill bit out of the work so it breaks off before continuing.
· Unplug drill and remove drill bit and chuck key from drill.
· Leave the drill clean and the area tidy.
· Switch off lights.
HAZARDS 

· Loose clothing may cause serious injury.
· Failing to remove chuck key may cause serious injury.
· Unsecured work may cause serious injury.
· Swarf longer than 100mm in length may cause serious injury.
· Electrocution.

Approved by Manager/Supervisor:                                                 



[image: image1.png]





Date:       /       /  2016
Diesel Force
6.56  Hand Held Drill
Revision: 0. 3rd July 2014

