SAFE WORK PRACTICE

ROTARY LIFT VEHICLE HOIST

All operators must be thoroughly trained in the use and care of hoist.

Only Authorised personnel are to operate lift.

A copy of the Operation & Maintenance Manual is to be kept in the pouch at all times

PPE (Minimum – check SWP- PPE for correct protection)

- Long hair secured

- Long sleeves buttoned

- Eye protection

- No loose clothing

- Gloves if needed

- Safety footwear

SAFETY INSTRUCTIONS

- Inspect the hoist daily.

- Prohibit unauthorised persons from area while lift is in use.

- Do not permit any one on hoist or inside vehicle while it is being raised or lowered.

- Always keep area around and under hoist free of tools, debris, grease and oil.

- NEVER overload lift. Capacity is shown on nameplate attached to hoist.

- Do not stand in front of vehicle while it is being positioned in lift bay.

- Keep all other plant and equipment at least 600mm clear of the hoist and the load.

- Do not run over lift arms or adaptors; this could damage hoist or vehicle.

- Do not go under vehicle if locking latches are not engaged.

- Do not block open or override self-closing lift controls.

- Do not remove or disable arm restraints.

- Remain clear of hoist when raising or lowering vehicle.

- Always use safety stands when removing or installing heavy components.

- Avoid excessive rocking of vehicle while on the hoist.

- Clear area if vehicle is in danger of falling.

- Remove tool trays, stands, etc. before lowering hoist.

- Release locking latches before lowering hoist.

- Position hoist arms and adapters to provide an unobstructed exit before removing vehicle from the hoist area.

HAZARDS

- Severe injury or death from incorrect positioning of vehicle.

- Damage to vehicle and/or hoist from incorrect use of hoist.

Approved by Manager/Supervisor:

Date:       /       /   2016