SAFE WORK PRACTICE

BRAKE CHECKING PROCEDURE

PPE (Minimum – check SWP- PPE for correct protection)

- Long hair secured

- Long sleeves buttoned

- Eye protection

- No loose clothing

OPERATION

Before proceeding, ensure vehicle is secure.

ENSURE ALL READINGS ARE RECORDED ON THE WORKSHOP BRAKE CHECK FORM.

WHEN CHECKING BRAKE DRUM TYPE BRAKES:

- ensure top and bottom shoes are checked and record the lower reading.

WHEN CHECKING A BRAKE DRUM:

- ensure you check the wear lip and record the depth reading.

WHEN CHECKING DISC TYPE BRAKES:

- ensure both or all the brake pad thicknesses are checked and record the lowest reading.

WHEN CHECKING A BRAKE DISC:

- ensure you check the wear lip on both sides of the disc and record the highest reading.

ALL STEERING AND BRAKE CHECKS ARE TO BE CARRIED OUT BY A FULLY QUALIFIED MECHANIC.

HAZARDS

- Crush injury from vehicle falling from jacks/stands

- Pinch points

Approved by Manager/Supervisor:

Date:       /       /   2017