[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

PEDESTAL DRILL
PPE (Minimum – check SWP- PPE for correct protection)

Long hair secured

Long sleeves buttoned

Eye protection

No loose clothing

No Gloves

OPERATION

· Turn lights on over work area

· Fit drill bit required to do the job, remembering to remove the chuck key before continuing

· Position work and secure with drill clamp or vice

· In no circumstance is work to be held by hand

· Select correct speed

· Proceed to work

· Do not allow swarf to exceed 100mm in length, remove it by lifting the drill bit out of the work so it breaks off before continuing

· Switch off and remove drill bit from work 

· Remove work

· Remove drill bit and chuck key from chuck

· Brush off drill and leave area tidy

· Switch off lights

 HAZARDS

Loose clothing may cause serious injury

Failing to remove chuck key may cause serious injury

Unsecured work may cause serious injury
Swarf longer than 100mm in length may cause serious injury
[image: image1.png]Approved by Manager/Supervisor:                                                 






Date:       /       /   2016
Dieselforce
6.6   Pedestal Drill
Revision: 1. 11th Jan. 2012

