[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

DANGER TAGS
Danger Tags are a red tag system designed to give personal protection to an individual working on a particular task or in a particular area.

1.
Supplies of danger tags shall be readily available.

2.
A danger tag shall be attached to the main isolating switch, valve or other positive isolating device on the unit, whenever a danger is posed to the individual from the operation of the machine, equipment or the flow of the energy source.

3.
It is the responsibility of the individual to identify any situation with the potential for danger and to ensure danger tags are used.

4.
Switches such as push buttons, emergency stops and control switches are not positive isolators and shall never be used as such.

5.
Each individual shall place their own danger tag on the main isolating apparatus, before commencing any work. If any uncertainty exists consult with your supervisor before proceeding.

6.
No individual shall operate any switch, valve or positive isolator when a danger tag is attached to it.

7.
A danger tag must have the name of the person placing the tag, what has been isolated, and the date recorded on it.

8.
An individual is only permitted to remove their own danger tag.

9.
Failure to obey clause 6 or 8 will result in the termination of the individual’s employment.

10.
At the completion of the task or at the end of the shift, the individual must remove his or her own danger tag. If the plant or equipment is still unsafe to operate, due to its condition, an out of service tag shall be completed and placed on the switch, valve or positive isolator, before removal of their danger tag.

11.
In circumstances where a person leaves the site without removing their danger tag, they will be contacted by the supervisor to return and remove their tag. Their return shall be at the expense of the individual.

12.
In the event that the individual who placed the danger tag and left it on is not able to be contacted, the following shall apply. The supervisor or manager, after a full investigation of the situation personally, may remove the danger tag when satisfied it is safe to do so.
Each step of the investigation must be documented with date and times and signed off.
13.
In situations where intermittent operation of equipment or plant is needed during maintenance work, a written variation to this procedure must be obtained from the relevant manager in the form of a JSA.
1 of 2

14. When Danger Tag is fitted, ensure that it is securely attached with a double knot or slip knot that will not untie itself in windy conditions.
15. When the job is completed and the danger tag is removed, it must be disposed of correctly.


Approved by Manager/Supervisor:                                                 

[image: image1.png]
















Date:       /       /   2016
2 of 2

Dieselforce
6.7  Danger Tags
Revision: 2. 14th March 2013

