[image: image1.png]et ONNEN OBNEN OBEEN OB OB OB Q..

DIESEL"FORCE

‘-D“-".V





SAFE WORK PRACTICE

MULTIPLE PERSON ISOLATION LOCK OUT
PPE (Minimum – check SWP- PPE for correct protection)
Long hair secured
Eye protection

Safety foot wear
OPERATION
· A competent person will be in charge of the procedure.
· A JSA 8.19 Isolation of Plant, must be used in conjunction with this SWP. 
· All participants must sign onto the JSA.
· If in doubt, contact your Supervisor

· Isolate machine at main lock out master switch. (If no switch, disconnect batteries).
· Remove earth lead, make safe and apply personal Danger Tag/s.

· All persons working on job to fit personal danger tags to multiple isolation clip. 
· Anyone not applying a Danger Tag or removing their Danger Tag shall not work on the machine.
· Only the person responsible for their tag may remove it 
(as per SWP 6.7 Danger Tags).
· Test machine at start point to check for correct isolation.

· Carry out work. 
· When the job is completed and personal tags removed, they must be disposed of correctly.

· All participants must sign off the JSA.

· Machine cannot be started until all persons have removed their tags from the multiple isolation clip. 
· If participants removing personal tags are not involved in start-up they must vacate site and report to Supervisor for alternative work.
· If assistance is required for fault finding/testing, then; 
· SWP 6.55 De-Isolation Machine Testing and JSA 8.20 must be used. 
HAZARDS

Un-intentional machine start-up causing injury/death.

Plant and equipment damage
Environmental Damage

[image: image1.png]Approved by Manager/Supervisor:                                                 







Date:    /     / 2016  
Dieselforce
6.9  Multiple Isolation Lockout Procedure
Revision: 2.6th February 2015

